# Trigonometric Approach to Forward Kinematics

## Summary:

This lesson introduces forward kinematics as the process of calculating where the end effector ends up given a set of joint angles. Students review the trigonometry needed (SOH-CAH-TOA) and apply it to derive the FK equations for a simplified 2-link planar arm and then in 3d. By the end of the lesson, students can calculate the predicted end-effector position for a given set of angles and verify it against the physical arm.

## Objective:

By the end of this lesson, students will be able to:

● Explain forward kinematics as calculating end-effector position from known joint angles

● Apply trigonometric principles (SOH-CAH-TOA) to a series of rotating joints

● Derive the FK equations for a simplified 2-link planar arm

● Calculate the end-effector position for a given set of joint angles

## Standards:

NGSS

## Resources & Materials:

Desmos Example

Slideshow

## Procedures:

Day 1: 2D Forward Kinematics

- Opening (3 min) — Display title slide; briefly tie back to the "how did I know which angles to use?" question from the lesson opener.

- Defining the arm's links (7 min) — Introduce L1 and L2 as the link lengths (e.g., 250mm each), emphasizing these are known constants because they're fixed, measured dimensions of the physical arm.

- Defining the joint angles (10 min) — Introduce θ1 and θ2, explaining each is measured relative to a vertical projection at its joint. Emphasize that although these angles are variable, they are always known because the encoder at each actuator measures its rotation — a callback to the encoders concept from Lesson 1.

- Setting up the problem (5 min) — Show the arm with origin (0,0) at the shoulder joint and (X,Y) marked as the unknown position of the end effector — the value the class is trying to solve for.

- Breaking position into components (10 min) — Show that the end-effector position is the sum of the horizontal and vertical displacement contributed by each joint (ΔX1, ΔY1, ΔX2), and that each displacement can be represented as a triangle.

- Deriving ΔX1 and ΔY1 (10 min) — Walk through using sin(θ1) and cos(θ1) with L1 as the hypotenuse to solve ΔX1 = L1sin(θ1) and ΔY1 = L1cos(θ1), plugging in θ1 = 45° to get 176.777mm for each.

- Deriving ΔX2 and ΔY2 (8 min) — Repeat the process using cos(θ2) and sin(θ2) with L2, showing ΔX2 = 250mm and ΔY2 = 0mm for θ2 = 45°.

- Combining the terms (5 min) — Show the final formula (X,Y) = (ΔX1+ΔX2, ΔY1+ΔY2) and complete the worked example: (X,Y) = (426.777, 176.777).

- Recap & wrap-up (2 min) — Show the "Completed Variables" slide as a clean summary of the full worked problem before dismissing.

Day 2: Extending to 3D with Base Rotation

- Recap (3 min) — Quickly review the 2D derivation and final formula from Day 1.

- Try It Yourself #1 (7 min) — Students solve for (X,Y) with θ1=30°, θ2=30°; reveal solution (341.506, 341.506).

- Try It Yourself #2 (6 min) — Students solve with θ1=60°, θ2=0°; reveal solution (433.013, 250). Use this one to highlight how a 0° angle simplifies the math.

- Introducing base rotation (8 min) — Present the "Forward Kinematics With Base Rotation" section: introduce R as the horizontal distance from the end effector to the base's center point, and show how R changes (R1 < R2) as the arm extends further from the base.

- θ3 and the circle (7 min) — Show that R forms a circle around the base's center point when rotated, and that θ3 (base rotation) is known via the base actuator's encoder — reinforcing the "always known" theme from Day 1.

- Problem-solving strategy (7 min) — Explain that the 3D problem splits into two 2D problems: first solving for R along the XZ plane (top-down view), then solving for X and Y along the arm's own plane (side view) — the exact same method from Day 1.

- Worked example problem (14 min) — Walk through the full example (θ1=45°, θ2=45°, θ3=45°) step by step: reuse the Day 1 result (X,Y)=(426.777,176.777) to get R=426.77mm, then apply Z=Rsin(θ3) and X=Rcos(θ3) to find the final 3D position (X,Y,Z) = (301.77, 176.78, 301.77).

- Practice Problem (7 min) — Students solve θ1=30°, θ2=30°, θ3=60°; reveal solution (187.5, 216.51, 324.76).

- Practice Problem 2 (6 min) — Students solve θ1=30°, θ2=90°, θ3=30°; reveal solution (295.75, -170.75, 91.51) — note the negative Y value as a discussion point (what does that mean physically?).

- Desmos Calculator demo (2 min) — Pull up the linked 3D Desmos calculator to visually confirm one of the solved problems, giving students a spatial/interactive check on their math before closing.

## Assessment:

Complete the example problems in the slideshow